- SOUTHLINKS COMMONS BUSINESS PARK

- INDUSTRIAL FLEX SPACE

- Dock high loading (Ramps available on request – part of TI)

- Bay sizes: Range TBD Square Feet (40’ x TBD’) or larger

- Clearance (bottom of joist): 18 feet clear height

- Overhead Doors: 8’x9’ manual insulated doors with bollards

- Exterior Glazing: Kawneer impact resistant glass double tinted grey, finish Anodized aluminum per architectural plans.

- Parking: percentage of office/warehouse

- Shell Design: Tilt wall construction with steel columns, bar joist and decking

- Window sills: not included

- Roofing: Single membrane Versaco or acceptable alternate (3.3” insulation (R20) with walk pads - white 60 mil.

- RWC internal with cleanout as required

- Condensate: Schedule 40 PVC to storm sewer or pre-treatment area

- Domestic Water Distribution: CPVC schedule 80 overhead (size per fixture count)

- Water potable: one 2” service water meter per building.

- Fire Protection: High Rack Level III - Steel pipe – design size determined by building area.

- Sanitary Sewer: Main 6” or larger PVC schedule 40 with clean out

- Electrical Service/Metering: (1) meter per tenant space

  - 208/120 V., 3-phase

- Exterior Lighting: (1) 250-watt LED building pack per (2) units, LED with concrete pole lights on 25-foot concrete poles and LED recess cans in soffit (1) per unit

- HVAC: DX roof system curbing (Carrier) on roof – mechanical/HVAC units furnished as part of tenant improvement construction.

- Low Voltage Systems: Telephone service conduits only at mechanical room, then each unit has telephone conduit provided from mechanical room underground to tenant space for future communication provider use (By Tenant). Fire alarm system EST typical.

- Hollow metal doors: Ceco or equal, frame and door, 3’ x 7’ with crash chains, threshold, weather stripping and keyed entry lock.

- Mechanical louver (3’ 4” x 3’ 4”) above man doors at rear of building

- Mechanical room located at the center rear of the building

- Landscaping: Per Development order.

- Building & Site Signage: TBD

- No generator – (Available upon request, as part of tenant improvement)

- Blinds: Included in shell.

- Caulking – Exterior tilt wall panel joints only (no interior)

- Exterior Paint – PPG Texture or equal (medium texture)